# Broadoak Community School Faculty of Science: AQA Double Science Module 10

### Choosing Fuses			Name _______________________

How to do it:

- Work out the current that the machine normally needs.

- Choose the smallest fuse value that will comfortably do the job. You usually get a choice of 3A, 5A or 13A.

To work out the current, remember that:

Amps = Watts  Volts		(in other words Current = Power  Voltage)

The voltage is usually 240V, so we can use   Amps = Watts  240V .

Unless the question tells you that the voltage is different

- What would happen if we used a 3A fuse for a fan heater that normally needs 8 Amps?

- Why is it a bad idea to use a 13A fuse for a bedside lamp that only needs 0.2A under normal conditions?

- An electric heater uses 4A when running. Would you fit it with a 3A, 5A or 13A fuse?

- A garden floodlight has a 480W bulb, and runs on 240V.
(a) How much current does it need? 	

(b) Would you fit it with a 3A, 5A or 13A fuse?

- An electric shaver uses 40W of power, and runs on 240V
(a) What current does it need? 	 

(b) Given a choice of 1A, 3A, 5A or 13A fuses, which would you use?	

Extra:

- Our main hall lighting rig contains 8 x 1,000W lamps, 6 x 1,200W, 6 x 600W, 12 x 650W and 15 x 500W. 
(a) When all the lamps are faded up, what is the total power? 	 	
 	

(b) Would a 150A fuse be enough to run the system?